TECHNICAL HIRING ASSIGNMENT

Senior Full-Stack AI / ML Engineer

Industrial Machine Learning · Deep Learning · Optimization · GenAI Agents

AI-Powered Cracked Gas Centrifugal Compressor — Reliability & Energy-Optimization Platform

|  | Mission Build one coherent, production-style application that ingests cracked gas compressor telemetry, models mechanical and performance degradation, and recommends operating and maintenance actions that jointly (a) extend run length and (b) reduce specific energy consumption — surfaced through a secured web platform with a multi-agent GenAI advisor grounded in real model and optimizer outputs. |
| --- | --- |

|  | Synthetic data notice The accompanying dataset is fully synthetic, generated from a physics-based simulation. Numbers, incidents, standards and documents are fictional and are provided solely for this assignment. They do not represent any real plant, company, or published standard. |
| --- | --- |

# At a glance

| Item | Detail |
| --- | --- |
| Target level | 2–4years. Senior candidates should go deeper on optimization, MLOps and agent design. |
| Submission window | 2 calendar days from receipt. |
| Deliverable | End-to-end running application (not isolated notebooks) + report + demo |
| Core stack (suggested) | Python, FastAPI, PostgreSQL (+ pgvector), a JS frontend (React/Next.js/Vue), Docker. |
| Mandatory disciplines | Classical ML, Deep Learning, mathematical Optimization, and GenAI (RAG + tools + agents) are ALL required — not optional. |
| Evaluation | Weighted rubric (see end) + a technical interview that will challenge your implementation. |
| AI tools | You may use AI coding assistants and open-source libraries. You must be able to explain every design choice. |

# Contents

| 0. Domain primer | 1. Business & engineering problem |
| --- | --- |
| 2. Provided dataset | 3. Data engineering, validation & EDA |
| 4. Physics-based feature engineering | 5. Health, degradation & anomaly modelling |
| 6. Machine-learning models | 7. Deep-learning sequence models |
| 8. Explainability | 9. Energy & run-length optimization |
| 10. Optimization implementation (API) | 11. Optimization & what-if scenarios |
| 12. Digital-twin what-if simulation | 13. GenAI multi-agent advisory system |
| 14. RAG over the knowledge base | 15. LLM tool / function calling |
| 16. Structured + unstructured fusion | 17. GenAI evaluation |
| 18. Safety guardrails | 19. Backend |
| 20. Database | 21. Authentication, authorization & RBAC |
| 22. Frontend | 23. Real-time streaming & alerting |
| 24. Model serving & MLOps | 25. Testing |
| 26. Docker & deployment | 27. Code quality |
| 28. Architecture & system design | 29. Production design & scaling |
| 30. Security | 31. Final technical report |
| 32. Git requirements | 33. Deliverables |
| • Evaluation criteria | • What we will look for |
| • Candidate instructions | • Demo video & interview follow-ups |

# Objective

You will build an AI platform for a critical piece of refinery / petrochemical rotating equipment: the cracked gas centrifugal compressor (CGC). The platform must turn raw compressor telemetry into decisions that operators and reliability engineers can act on, with two explicit and sometimes competing goals:

- Increase run length — maximise the operating hours between interventions (online washes, bearing work, shutdowns) without ever breaching a protective limit.

- Reduce energy consumption — minimise Specific Energy Consumption (kWh per tonne of gas), which on a steam-driven machine translates directly into steam and money.

The point of the assignment is not merely to produce a working app. It is to demonstrate that you understand WHY and WHEN each technique (classical ML, deep sequence models, optimization, RAG, tool-calling, agents) is the right tool, that you can connect them into a safe, explainable system, and that you respect the fact that this is safety-critical rotating equipment where a wrong number can trip a multi-megawatt machine.

|  | The bar A strong submission reads like a small, real reliability-and-energy product: physics-aware features, honestly-validated models, an optimizer with genuine engineering constraints, an agent that never invents numbers, and an interface a plant engineer could actually use behind a login. |
| --- | --- |

# 0.  Domain primer (read this first)

You are not expected to be a rotating-equipment engineer, so here is the minimum you need. The knowledge-base documents in the dataset go further and are also the corpus for your RAG task.

## 0.1  What the machine is

A cracked gas compressor is a large multi-stage centrifugal compressor in an ethylene plant. It takes cracked (pyrolysis) gas at near-atmospheric pressure and compresses it — here across five stages with interstage cooling — to ~36–37 bar so the downstream separation train can recover ethylene, propylene and other products. The trains in this dataset are driven by extraction-condensing steam turbines and draw tens of megawatts, so they are usually the single largest energy consumer in the plant.

## 0.2  The two things that go wrong

- Fouling. Cracked gas contains dienes that polymerise and deposit on the impellers and diffusers. Deposits narrow the flow path, lower polytropic efficiency, raise discharge temperature and specific energy, and eat into surge margin. Fouling is the dominant, controllable driver of both lost run length and wasted energy. It is fought with antifoulant dosing, wash-oil injection and periodic online washes (which partially restore efficiency).

- Mechanical degradation. Bearings wear, vibration rises, seals and lube systems drift. These show up in vibration and bearing-temperature trends and, if missed, end in an unplanned high-vibration trip — far costlier than a planned intervention.

## 0.3  The physics you will actually use

You must compute performance from raw signals rather than being handed it. Polytropic head per stage:

H_p = Z · (R/MW) · T_suct · (n/(n-1)) · [ (P_disch/P_suct)^((n-1)/n) − 1 ] Discharge temperature : T_disch = T_suct · (P_disch/P_suct)^((k-1)/(k·η_poly)) Shaft power : W ≈ (ṁ · ΣH_p) / η_mech Specific energy (KPI) : SEC = shaft_power_kw / throughput_tph [kWh/tonne] Surge margin : distance (%) of operating flow above the surge-limit flow

From the per-stage pressures and temperatures, gas molecular weight and flow in the telemetry, you can back out polytropic efficiency, head and SEC, and cross-check a measured discharge temperature against the value the physics predicts — which is how you catch a drifting sensor.

|  | Run length vs energy — the core tension A fouled machine wastes energy, so washing sooner saves energy — but each wash costs production and shortens the effective run. Push run length too far and efficiency (and surge margin) degrade until you risk an unplanned event. Your optimizer has to reason about this trade-off explicitly, not optimise one side blind to the other. |
| --- | --- |

# 1.  Business & engineering problem

Plant management and the reliability team want a system that can answer, with evidence:

- How is each compressor train degrading right now, mechanically and in performance?

- How much energy are we wasting to fouling, and what is it costing?

- When should we schedule the next online wash to best trade energy against lost run length?

- What operating point (speed, recycle, throughput) minimises energy while holding every safety limit?

- How long until a protective limit or wash trigger is reached (remaining useful life)?

- Is anything anomalous — an incipient bearing fault, a surge excursion, a bad sensor?

- Can an assistant explain each recommendation to an operator, grounded in our own standards and live numbers?

Your platform must answer these using the actual outputs of your models and optimizer — never by an LLM guessing.

# 2.  Provided dataset

A synthetic, physics-generated dataset is supplied (see DATA_DICTIONARY.md). Two 5-stage steam-turbine-driven trains (CGC-100A, CGC-200B) at hourly resolution over 24 months — 35,040 telemetry rows.

| File |  | What it is |
| --- | --- | --- |
| timeseries/compressor_telemetry.csv |  | Main sensor & operating telemetry — your model INPUT (45 columns: per-stage P/T, flow, speed, power, vibration, bearing temps, lube oil, surge margin, recycle valve, feed composition, mitigation actions). |
| timeseries/ground_truth_reference.csv |  | Simulation LATENT truth (fouling, efficiency, health, RUL). Validation/target construction only — NOT a model input. |
| reference/events.csv |  | Washes, surge excursions, a trip, unscheduled maintenance, sensor faults. |
| reference/assets.csv |  | Design specs and protective limits per train. |
| reference/compressor_curves.csv |  | Performance map / surge-line reference. |
| reference/feed_slate_daily.csv |  | Daily feed composition (ethane / mixed / naphtha). |
| knowledge_base/*.md (7 docs) |  | Engineering standards, procedures, energy policy, incident log — the RAG corpus. |
|  | Label-leakage trap (deliberate) The *_true columns are latent simulation states that do not exist in a real plant — fouling is never directly measured. Use them only to validate models and to build honest targets. Using any *_true column as a feature is leakage and is treated as a serious, score-reducing error. Demonstrating that you understand this is part of the test. |  |

You may add engineered features and external assumptions; document them. You may not swap in a different dataset — the tasks below assume this one.

# 3.  Data engineering, validation & EDA

Build a reproducible preprocessing pipeline (not throwaway notebook cells) that addresses:

- Missing values and the multi-day gaps left by the outage (do not forward-fill across a shutdown as if the machine were running).

- Duplicate / out-of-order timestamps and correct time indexing per train.

- Physically-impossible values and sensor spikes; a stuck (frozen) transmitter; a drifting transmitter.

- Sensor validation using physics cross-checks (e.g., measured vs. polytropic-predicted discharge temperature) — flag BAD data, do not silently model it.

- Type coercion, resampling / alignment, and per-train scaling that is fit on training data only.

- Feature engineering: pressure ratios, computed polytropic head & efficiency, SEC, surge margin trends, run-hours features, rolling statistics, rate-of-change, feed-slate encodings, ambient/CW normalisation.

## EDA — identify and visualise

- Degradation cycles: efficiency / SEC drift within a run and the recovery at each wash.

- The effect of feed slate (ethane vs naphtha) on fouling rate and duty.

- Seasonality from ambient & cooling-water temperature.

- The injected anomalies: surge cluster, bearing-wear ramp → trip, sensor drift, stuck sensor.

- Differences between the two trains, and correlations among key variables.

|  | Deliverable A reproducible pipeline (e.g. src/data/, src/features/) plus an EDA notebook or report. Reproducibility and correct time-series hygiene matter more than the number of charts. |
| --- | --- |

# 4.  Physics-based feature engineering (digital-twin foundation)

This section is what distinguishes this assignment from a generic tabular problem, and is weighted accordingly. Implement a small, tested physics module that computes, per stage and overall:

- Pressure ratio (per stage and overall).

- Polytropic head and polytropic efficiency from P, T, MW and flow.

- Specific energy consumption and an estimate of the excess energy attributable to fouling (vs the clean baseline at comparable flow).

- Surge margin relative to the supplied surge-line reference.

- A physics-consistency residual: measured discharge temperature minus polytropic-predicted discharge temperature (your sensor-fault detector).

|  | Why this matters A model that predicts degradation from raw tags alone is brittle. Physics-derived features are transferable across trains and operating points, make the model explainable to engineers, and let the GenAI layer answer 'how much energy is fouling costing us' with a real number. You will be asked in interview to derive the polytropic relations. |
| --- | --- |

# 5.  Health, degradation & anomaly modeling

## 5.1  Performance-degradation forecasting

Forecast the near-future trajectory of a degradation KPI you define (e.g. polytropic efficiency or fouling-adjusted SEC) per train, so the wash trigger can be anticipated rather than merely detected. Use proper time-series validation (walk-forward), and state your forecast horizon.

## 5.2  Remaining useful life (RUL)

Estimate two RULs and take the shorter as the planning driver: (a) performance RUL — hours until a wash trigger from RS-CGC-03 is forecast to be met; (b) mechanical RUL — hours until a vibration or bearing-temperature limit is forecast to be reached. Quantify uncertainty, not just a point estimate.

## 5.3  Anomaly detection

Detect the abnormal conditions in the data (surge excursions, the bearing-wear ramp, sensor drift/stuck). Combine at least one unsupervised method (e.g. residual-based, isolation forest, autoencoder) with a physics/rule check. Report precision/recall against events.csv and discuss false alarms — a nuisance-tripping detector is useless to an operator.

# 6.  Machine-learning models

Train and compare at least two approaches for your primary degradation/forecasting target — a transparent baseline and a stronger model:

- Baseline: linear / regularised regression, or a naive/seasonal persistence forecast.

- Advanced: gradient boosting (XGBoost / LightGBM) or Random Forest, or another justified choice.

Document target definition, features, the train/validation/test split and WHY it is valid for time-series (no shuffling across time; no future leakage), feature engineering, model selection, hyper-parameter tuning and overfitting control. Report a comparison:

| Model |  | MAE | RMSE | MAPE / WAPE | Train time | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline |  |  |  |  |  |  |
| Model 1 |  |  |  |  |  |  |
| Model 2 |  |  |  |  |  |  |
|  | Mandatory Explain precisely how you prevented data leakage — including leakage from the *_true columns, from rolling features that peek forward, and from fitting scalers on the full series. |  |  |  |  |  |

# 7.  Deep-learning sequence models

Build a deep model suited to multivariate compressor time-series — LSTM, GRU, temporal convolutional network, or a Transformer-style encoder (you need not implement attention from scratch). Frame it as multi-step forecasting of your degradation KPI and/or RUL, or as sequence anomaly detection.

Document input windowing, architecture, layers, activations, loss, optimizer, learning rate, batch size, epochs, and validation strategy. Address overfitting (dropout, early stopping, regularisation, weight decay).

|  | The question that earns marks Compare the deep model against your classical ML model and answer honestly: did deep learning actually improve the result on THIS dataset? If not, would you still ship it, and why or why not? A candidate who deploys an LSTM that loses to XGBoost without justification scores worse than one who chooses the simpler model deliberately. |
| --- | --- |

# 8.  Explainability

Explain what drives your predictions using SHAP, permutation importance, partial dependence, or a comparable method, and reconcile it with the physics. The system should be able to answer, for an engineer:

- What is driving the rising specific energy on CGC-100A this week?

- Which signals most influenced the RUL estimate for CGC-200B before the trip?

- Is the model relying on physically sensible variables, or on a leak / artefact?

# 9.  Energy & run-length optimization

This is a core, heavily-weighted section. Using your forecasts as inputs, formulate and solve optimization problems that operationalise the two objectives. Implement at least the first; strong candidates implement both and connect them.

## 9.1  Operating-point energy optimization

- Decision variables: setpoints you can move — e.g. speed / load split across the train, recycle (anti-surge) valve position, antifoulant dose, throughput within an allowed band.

- Objective: minimise specific energy consumption (equivalently shaft power / steam cost) for the required throughput.

- Constraints: surge margin ≥ 8% (hard safety limit), vibration and bearing-temperature limits, speed within min-governor and trip limits, discharge pressure/temperature limits, minimum throughput, antifoulant ≤ 22 ppm.

## 9.2  Wash-scheduling / run-length optimization

- Decision: when to perform the next online wash (and optionally the antifoulant policy) over a planning horizon.

- Objective: minimise total expected cost = energy cost of running fouled + wash cost (lost production) + risk/penalty of breaching a limit — i.e. the explicit energy-vs-run-length trade-off. A multi-objective / Pareto treatment is welcome.

- Constraints: efficiency/margin floors, minimum time between washes, planning-horizon demand.

Use an appropriate solver (SciPy, PuLP, Pyomo, OR-Tools, or a documented metaheuristic). Justify the formulation (LP / MILP / NLP / dynamic) and the solver choice. Clearly state decision variables, objective and constraints for each problem.

# 10.  Optimization implementation (API)

Expose the optimizer behind the backend. Example contract:

POST /api/optimization/operating-point { "train_id": "CGC-100A", "required_throughput_tph": 255, "min_surge_margin_pct": 8, "horizon_hours": 24 } 200 OK { "status": "optimal", "baseline_sec_kwh_per_t": 108.4, "optimized_sec_kwh_per_t": 101.7, "energy_saving_pct": 6.2, "setpoints": { "speed_rpm": 10480, "recycle_valve_pct": 4.0, "antifoulant_ppm": 12 }, "binding_constraints": ["surge_margin>=8"], "explanation_inputs": { ... } }

Handle infeasible requests explicitly (e.g. required throughput incompatible with the surge constraint) and return a clear, non-crashing response.

# 11.  Optimization & what-if scenarios

Demonstrate how the solution responds to changing conditions, and explain the mechanism behind each change:

| Scenario | Condition | What we expect you to discuss |
| --- | --- | --- |
| S1 | Base case, mid-run fouling | Reference energy and setpoints. |
| S2 | Switch to heavy (naphtha) feed | Higher duty & fouling rate; effect on wash timing and energy. |
| S3 | Cooling-water temperature +6 °C (summer) | Interstage temps up, duty up; where energy is lost. |
| S4 | Throughput demand +8% | Speed/margin response; is any limit binding? |
| S5 | Tighter surge constraint (min margin 12%) | Energy penalty of extra safety margin. |
| S6 | Steam price +25% | How the wash-vs-run trade-off shifts. |

# 12.  Digital-twin what-if simulation

Wrap your physics module + degradation model into a lightweight simulator the user can drive: given a starting state and a set of assumptions (feed slate, ambient, throughput, wash date), project SEC, surge margin, efficiency and RUL forward over a chosen horizon. This is the engine behind the scenarios in §11 and the what-if questions the agent will ask in §13. It need not be a high-fidelity CFD model — it must be physically consistent and clearly bounded.

# 13.  GenAI multi-agent advisory system

Upgrade the usual single chatbot into a small orchestrated multi-agent system. At minimum implement an orchestrator plus specialised agents; you may merge roles if you justify it, but a single monolithic prompt is not sufficient.

- Reliability Agent — degradation, anomalies, RUL, maintenance timing.

- Energy Agent — SEC, energy losses, operating-point and wash-timing optimization.

- Operations Advisor — translates model/optimizer outputs into plain, safe operator guidance, grounded in the standards.

- Orchestrator — routes a user question to the right agent(s)/tools, composes the final answer, and enforces the guardrails in §18.

The assistant should answer questions such as: 'Which train is wasting the most energy and why?', 'When should we wash CGC-100A and what will it save?', 'Explain the current operating-point recommendation', 'Is CGC-200B showing any early bearing problem?', 'What happens to energy if we defer the wash two weeks?' — always using real outputs from your backend.

# 14.  Retrieval-augmented generation over the knowledge base

Implement RAG over the seven knowledge-base documents (standards, procedures, energy policy, incident log). Pipeline: extract → chunk → embed → store → retrieve → generate with cited sources.

Documents → Text extraction → Chunking → Embeddings → Vector store → Retrieval → LLM → Answer + cited sources

Use PostgreSQL + pgvector, Qdrant, Chroma, FAISS or similar, and justify the choice. The assistant must answer policy questions ('what triggers an online wash?', 'what is the minimum surge margin?', 'what was the root cause of the CGC-200B trip?') with citations to the specific document.

# 15.  LLM tool / function calling

Expose backend capabilities as tools the agents call; numeric results must come from these tools, never from the model's imagination:

| get_train_health(train_id) -> health, latest KPIs, anomaly flags get_degradation_forecast(train_id, h) -> efficiency/SEC trajectory + interval get_rul(train_id) -> performance RUL, mechanical RUL, uncertainty compute_energy_loss(train_id) -> excess kWh/t & cost attributable to fouling optimize_operating_point(train_id, ...) -> setpoints, energy saving, binding constraints optimize_wash_schedule(train_id, ...) -> recommended wash date, cost breakdown simulate_whatif(train_id, assumptions) -> projected SEC / margin / RUL search_knowledge_base(query) -> policy passages + sources |  |
| --- | --- |
|  | Grounding rule If a user asks 'what should the recommended order/setpoint be', the number comes from the optimizer tool. If the LLM's narrative and the tool disagree, the tool wins and the discrepancy must be caught. You will be asked in interview how you guarantee this. |

# 16.  Structured + unstructured + telemetry fusion

Show the assistant combining unstructured policy (RAG) with structured model/optimizer output and live telemetry in a single answer. Example: 'According to our wash program, is CGC-100A due for a wash, and what will it save?' should retrieve the RS-CGC-03 trigger criteria, evaluate them against the train's current computed efficiency/SEC, call the wash-schedule optimizer, and present a cited, numeric recommendation.

# 17.  GenAI evaluation

Provide an evaluation set of at least 15 questions spanning RAG-only, tool-only, and fused questions, plus at least 2 adversarial / unsafe prompts. For each, document: question, expected answer, actual answer, tools called, retrieved sources, and correct/incorrect. Report groundedness, tool-call correctness, retrieval quality, hallucination rate and refusal behaviour, and describe concrete improvements.

# 18.  Safety guardrails (safety-critical)

This is rotating equipment; a confident wrong number is dangerous. Implement and document guardrails:

- No fabricated numbers — every quantitative claim must be traceable to a tool result.

- Hard constraints (surge margin ≥ 8%, vibration/temperature/speed limits) can never be recommended away by the agent; a suggestion that would breach them must be rejected with an explanation.

- Tool-vs-narrative consistency checks; refuse or escalate on conflict.

- Prompt-injection defence for both user input and retrieved documents (a malicious KB chunk must not be able to trigger a tool or override a limit).

- Scope & authorisation on tools (see §21) — a read-only role cannot trigger a state-changing action.

# 19.  Backend

FastAPI (preferred) or Django REST. Provide request validation, typed response schemas, error handling, structured logging, config management and OpenAPI docs. Suggested surface (extend as needed):

POST /api/auth/login GET /api/trains GET /api/trains/{id}/telemetry GET /api/trains/{id}/health GET /api/trains/{id}/forecast GET /api/trains/{id}/rul GET /api/trains/{id}/anomalies POST /api/optimization/operating-point POST /api/optimization/wash POST /api/simulate/whatif POST /api/documents/upload GET /api/documents POST /api/chat (SSE/stream) GET /api/health

# 20.  Database

PostgreSQL (preferred), with pgvector if you use it for RAG. Design tables such as: trains/assets, telemetry (consider a time-series-friendly layout / partitioning / hypertable), features, forecasts, rul_estimates, anomalies, optimization_runs, optimization_results, documents, document_chunks (+ embeddings), chat_sessions, chat_messages, users, roles, audit_log. Provide the schema / ER diagram and justify key indexes (time+train composite, vector index).

# 21.  Authentication, authorization & access control

The platform sits behind a login. Implement real authentication and role-based access control — this is an explicit requirement, not optional.

- Login with securely hashed credentials (e.g. bcrypt/argon2) and token-based sessions (JWT or equivalent), including expiry/refresh.

- At least three roles with different rights: Operator (read + acknowledge alerts), Reliability/Energy Engineer (read + run optimizations & simulations), Admin (user management + document upload).

- Authorization enforced at the API layer AND reflected in the agent's tools — a state-changing tool must check the caller's role.

- An audit log of logins, optimization runs and any action a recommendation drives.

|  | Minimum bar A working login page, protected routes/APIs, and at least one endpoint/tool that is correctly forbidden to a lower-privilege role, demonstrated in the video. |
| --- | --- |

# 22.  Frontend

A professional, functional web interface (React / Next.js / Vue or equivalent) behind the login. Suggested views:

- Login page and role-aware navigation.

- Fleet overview: both trains, health, current SEC vs target, active alerts, RUL summary.

- Train detail: historical & forecast KPIs, computed efficiency/head/SEC, per-stage view, feature importance, current recommendation.

- Energy & optimization: run the operating-point and wash-schedule optimizers; show savings, setpoints and binding constraints; scenario/what-if controls.

- Reliability: vibration/bearing trends, anomaly timeline, RUL with uncertainty.

- Assistant: chat with the multi-agent advisor — streamed answers, visible tool calls, cited sources, loading and error states.

# 23.  Real-time streaming & alerting

Demonstrate a near-real-time path: replay the historical telemetry as a live stream (a simple replayer is fine) into the platform, updating health/anomaly state and raising alerts when a limit or wash trigger is crossed. Streamed chat responses (SSE/WebSocket) and a live-updating dashboard tile are sufficient to show the pattern; full production streaming infrastructure is not required.

# 24.  Model serving & MLOps

Do not retrain on every request. Train → persist versioned artifacts → load at startup → serve. In the report, explain model versioning, a registry, retraining triggers, rollback, monitoring and — importantly for this domain — data-drift and concept-drift detection (feed-slate and seasonal shifts will move the input distribution).

# 25.  Testing

- Backend: at least 6 meaningful API tests including an auth/authorization test (forbidden role).

- Data/physics: tests for the physics module (known-input polytropic head/efficiency), feature generation, and leakage guards.

- ML/DL: preprocessing, prediction shape/range, and a walk-forward split correctness test.

- Optimization: constraint satisfaction, a known-optimum sanity case, and infeasible-scenario handling.

- GenAI: retrieval, tool-call correctness, the grounding rule, and a prompt-injection test.

# 26.  Docker & deployment

Provide Dockerfile(s), docker-compose.yml and .env.example. Ideally the whole stack (db + backend + frontend + vector store) starts with a single command:

docker compose up

Never commit secrets or API keys.

# 27.  Code quality

Clear project structure, meaningful naming, type hints, separation of concerns, error handling, reusability, logging, configuration management and documentation. Do not put the whole application in one or two files.

# 28.  Architecture & system design

Provide an architecture diagram covering the major components and the flow from telemetry to decision. A representative shape:

┌────────────┐ live/replay ─▶ │ Frontend │ ◀─ auth (login, RBAC) telemetry └─────┬──────┘ ▼ ┌────────────┐ ┌──────────────────────────┐ │ FastAPI │───▶│ Physics module / features│ └─────┬──────┘ └──────────────────────────┘ ┌──────────────┼───────────────┬───────────────┐ ▼ ▼ ▼ ▼ ┌──────────┐ ┌────────────┐ ┌───────────┐ ┌──────────────┐ │ ML / DL │ │ Optimizer │ │ Anomaly / │ │ GenAI agents │ │ forecast │ │ (energy / │ │ RUL │ │ + RAG + tools│ └────┬─────┘ │ wash) │ └─────┬─────┘ └──────┬───────┘ │ └─────┬──────┘ │ │ └──────────────┴───────────────┴───────────────┘ ▼ ▼ ┌───────────────┐ ┌───────────────┐ │ PostgreSQL │ │ Vector store │ │ (+ timeseries)│ │ (pgvector/…) │ └───────────────┘ └───────────────┘

# 29.  Production design & scaling

In the README, explain how you would scale to: many trains and years of high-frequency telemetry; concurrent engineers; long-running optimization jobs; a large document collection; and high-volume LLM requests. Discuss where you would use background workers / queues (Celery, etc.), caching (Redis), object storage, a time-series database, database indexing/partitioning and vector-DB scaling. Implementation of all of these is not required — reasoning is.

# 30.  Security

- API authentication & authorization (see §21); least-privilege tools.

- Input and file-upload validation; safe document ingestion for RAG.

- Prompt-injection defence for user input and retrieved content.

- Secrets / API-key management; no secrets in the repo.

- Rate limiting and abuse protection on chat and optimization endpoints.

- A note on OT/plant context: an advisory system must not be able to write setpoints to real control systems without human authorisation — discuss this boundary.

# 31.  Final technical report

Include TECHNICAL_DECISIONS.md answering:

| Area | Question to answer |
| --- | --- |
| Physics / features | How did you compute efficiency/SEC/surge margin, and how did physics features help? |
| ML | Why the final classical model? |
| DL | Did deep learning beat classical ML here — and would you still use it? |
| Degradation / RUL | How did you define the target and validate without leakage? |
| Optimization | Why this formulation and solver? How is the energy-vs-run-length trade-off modelled? |
| GenAI | Why RAG + tools + agents rather than fine-tuning or a single prompt? |
| Vector search | Why this vector store? |
| Safety | How do you guarantee the agent never recommends an unsafe or fabricated action? |
| Architecture | What would you change for production? |
| Limitations | What are the current limitations? |

# 32.  Git requirements

Use Git with meaningful, incremental commits. A single final commit containing the whole project is discouraged. Example history:

feat: data validation & physics feature module feat: degradation forecasting (baseline + gradient boosting) feat: LSTM sequence model + comparison feat: anomaly detection & RUL feat: energy & wash-schedule optimization + API feat: RAG over knowledge base feat: multi-agent advisor with tool calling + guardrails feat: auth, RBAC & audit log feat: dashboard, optimization & assistant UI test: physics, optimization, and GenAI grounding tests

# 33.  Deliverables

- Git repository with README and architecture + ER diagrams.

- Reproducible data + physics-feature pipeline.

- ML and DL training pipelines with saved, versioned models (or reproducible training instructions).

- Degradation, RUL and anomaly components.

- Energy and wash-schedule optimization + API and scenario results.

- Digital-twin what-if simulator.

- RAG implementation and multi-agent tool-calling assistant with guardrails.

- Backend APIs, authentication/RBAC and audit log.

- Frontend behind login.

- Real-time replay/alerting demonstration.

- Tests and the GenAI evaluation results.

- TECHNICAL_DECISIONS.md, .env.example and Docker configuration.

- A 10–15 minute demo video (see below).

# Evaluation criteria

| Area | Weight |
| --- | --- |
| Physics-aware data & feature engineering | 12% |
| ML model building & honest evaluation | 12% |
| Deep learning & the ML-vs-DL judgement | 8% |
| Degradation, RUL & anomaly detection | 10% |
| Optimization & mathematical modelling (energy + run length) | 16% |
| GenAI: RAG, tool calling, multi-agent & safety guardrails | 15% |
| Backend, database & SQL | 8% |
| Authentication / RBAC / security thinking | 5% |
| Frontend & real-time UX | 5% |
| Testing & code quality | 5% |
| Architecture, MLOps & production/scaling thinking | 4% |
| Total | 100% |

# What we will look for

- Build — a real end-to-end application, not isolated notebooks.

- Understand — explain the compression thermodynamics and the maths behind your ML, DL and optimization choices.

- Judge — know when a simple model beats a deep one, and choose deliberately.

- Optimize — formulate the energy-vs-run-length trade-off with honest objectives and real constraints.

- Integrate — wire models, physics and optimizer into a production-style API and UI.

- Use GenAI correctly — RAG, tools and agents that never invent numbers and never recommend unsafe actions.

- Engineer safely — auth, guardrails, testing, versioning, drift and failure handling appropriate to safety-critical equipment.

# Candidate instructions

You may use AI coding assistants, LLMs, open-source libraries and public documentation. You must be able to explain and modify your implementation during the interview. We may ask you to change an optimization constraint, add an LLM tool, modify the RAG pipeline, explain a SQL query, debug an API, or derive the polytropic relations on the spot.

## Required: demo video (10–15 min)

Show the full flow end-to-end:

Log in → view fleet & train health → show computed efficiency/SEC and a degradation forecast → run the energy optimizer and a wash-schedule optimization → run a what-if scenario → ask the assistant a fused question → show the tool call, the cited sources and the grounded number → demonstrate a role being correctly forbidden an action.

## Technical-interview follow-ups (representative)

- Your degradation forecast lags right after a wash. What would you change?

- Add a hard constraint that speed may not exceed 10,700 rpm and re-solve — what moves?

- Your LSTM beats XGBoost on CGC-100A but not CGC-200B. Why, and which do you ship?

- The assistant says the optimal wash is in 5 days but the optimizer tool returns 12. How do you prevent and detect this?

- A retrieved policy chunk contains an instruction to 'ignore the surge limit'. What happens in your system?

- How did you make sure fouling ground-truth never leaked into your features?

The objective is to evaluate engineering and problem-solving ability — not the volume of code produced.